IO Place Revision

./flow.tcl -interactive

set ::env(FP_IO_MODE) 2  // change the IOs to equidistant in mode 2

Then, run floorplan and load it in magic (as mentioned in section2)

Clone custom inverter standard cell design

//make sure is now in the right directory ‘openlane’

git clone https://github.com/nickson-jose/vsdstdcelldesign // clone the design from github repository

//find out that it already exist

//Go to the right directory

cd vsdstdcelldesign

//copy

cp /home/beaver/Desktop/work/tools/openlane_working_dir/pdks/sky130A/libs.tech/magic/sky130A.tech

//check if everything exist

ls

//load the custom inverter in magic

Magic -T sky130A.tech sky130_inv.mag

Press ‘s’ the second time to see which part the selected part is connected to

As mentioned in section2, use what to identify pmos and nmos

Spice extraction of inverter in magic

pwd

extract all //extraction command to extract to .ext format

ext2spice cthresh 0 rthresh0 // extracts the parasitic information

ext2spice // converting ext to spice

//check if the sky130_inv.spice now exist

/’can’t directly use vim to edit sky130_inv.spice, copy it to a new file

cp sky130_inv.spice temp.spice

//open the file

sudo vim temp.spice

/’go to the very bottom, press ‘i’ to edit it

When finish, press ‘esc’ to exit writing mode

Then, press ‘:wq’ to save and exit

//edit all the information and make it finally shows like this

//generate the graph

ngspice temp.spice

//At the end, to get the plot:

Plot y vs time a

Rise transition time = Time taken for output to rise to 80% - Time taken for output to rise to 20%

(value of 80% is 0.8*3.3 = 2.64)

(value of 20% is 0.2*3.3 = 0.66)

= 4.04167ns - 4ns = 0.04167ns

Rise cell delay = time taken for output to rise to 50% - time taken for input to fall to 50%

(50% is 0.5*3.3 = 1.65)

=4.05424 - 4.04989 = 0.00435

#### Find problem in the DRC section of the old magic tech file for the skywater process and fix them

// Change to home directory

cd

//download

wget http://opencircuitdesign.com/open_pdks/archive/drc_tests.tgz

tar xfz drc_tests.tgz //extract the files

cd drc_tests

ls -al

vi .magicrc //doesn’t have gvim, used vi to view the file

magic -d XR &

Poly

Load poly//load the poly layout

Edit the file to correct it

Press ‘/poly’ to find our target

Press ‘i’ to edit the file

After editing, go back to tkcon, run the command and see the changes

Difftap

nwell

//have to select the nwell.4 before checking